NEW 2022, 4.25-CYD, FRONT END LOADER BID SPECIFICATIONS

The Crawford County Road Commission will receive sealed bids until 4:00 p.m. (for a bid opening during the regular Board meeting starting at 4:30 p.m.) On Thursday March 3rd, 2022, at their Grayling office located on 500 Huron Street Grayling, MI 49738, for furnishing and delivering via a Five (5) year lease or 4,500 hours, one (1) articulated front end loader according to the following specifications:

GENERAL:  It is the desire of the Crawford County Road Commission to acquire the equipment by utilizing the bidders lease program. Length of lease shall be Five (5) years. The equipment required in the following specifications shall be a new, currently advertised standard production front-end loader, furnished with all necessary operating accessories customarily furnished.  Enclose descriptive literature and manufacturer's specifications with each bid and complete specifications sheet attached for each model of each unit bid.

ENGINE:  The engine shall be of the diesel type, fully equipped with all necessary accessories, including a fuel priming pump and full flow type oil filter.  The SAE flywheel horsepower shall be at least 225 horsepower with the following equipment included: Blower fan and adequate cooling capacity, water pump, lubricating oil pump, fuel pump, muffler, air compressor and alternator.  The engine air cleaner shall be the dry element type with screened pre-cleaner or equal.

ELECTRICAL SYSTEM: Machine shall be equipped with 24-volt heavy-duty electrical starting, operating and lighting system.  95 amp. (Min.) Delco alternator. Bidder shall provide a 12-volt 20-amp converter to accommodate additional lights and County Radio.

TRANSMISSION: The transmission shall be a fully automatic, planetary type, power shift capable of making repeated speed and directional changes at full torque. The automatic transmission shall have a minimum of four (4) speeds forward and four (4) speeds reverse and be actuated by a single lever. Provide adequate cooling capacity for the transmission oil.

BRAKES:  Power disc brakes on all four wheels. Two brake pedals: Right pedal brakes only; left pedal brakes while neutralizing the transmission. Brakes shall be sealed from water, dust and mud. Parking brake shall be mounted on transmission output shaft. Loader shall be equipped with an emergency brake system which, when air pressure drops, applies automatically to bring the machine to a controlled stop.

AXLES: Front axle shall be equipped with a limited slip differential or a torque-proportioning differential.  Rear axle shall provide for total oscillation of a least 13 degrees.

FINAL DRIVES:  Four-wheel drive with planetary reduction in each wheel.

Page 1

New 2022, 4.25 cyd, front end loader bid specifications continued;

TIRES AND WHEELS: All four tires are to be tubeless traction type Radial tire, sized 23.5 X R25 Michelin tires. Rear tires to be loaded for weight and stabilization.

STEERING:  Power steering shall be accomplished through use of an articulated frame actuated by two hydraulic cylinders.  Minimum steering angle in each direction shall be 40 degrees. Minimum turning radius of 19 feet.

LOADER HYDRAULIC SYSTEM:  The hydraulic system shall be fully filtered and enclosed with no vents or breathers in the tank.  All valves shall be in the tank to keep out dirt and other contaminants.  Hydraulic controls shall provide fast, constant control speed regardless of engine RPM.  Hydraulic system shall let operator use more than one control without decrease in control response speed.  The hydraulic pump shall provide a minimum 70-gpm output @ 2200 rpm.

BUCKET SPECIFICATIONS AND CONTROLS: The machine shall be equipped with a 4.25 cubic yard SAE heaped general-purpose bucket.  The cutting edge shall be a flat, reversible bolt on type with interchangeable ends.  Maximum width of 112".  Minimum static tipping load straight shall be a minimum of 26,000 pounds without tire ballast or counterweights.  Minimum static tipping load at full turn shall be a minimum of 23,000 pounds without tire ballast or counterweights.  The bucket controls shall be as follows: Mono lever operated which activates a lift circuit with four positions; raise, hold, lower, float and a tilt circuit with three positions: roll back, hold and dump.  Include an automatic bucket leveler.  Lift height at hinge pin of bucket shall be a minimum 13'-0".

CAB AND OPERATORS COMPARTMENT:  Loader shall be equipped with a fully enclosed rollover protective structure (ROPS) that meets current MIOSHA regulations at the time of purchase.  The cab shall be equipped with the following accessories:

(a) 	Heavy-duty heater with variable speed fan.

(b) 	Heavy-duty defroster with fan.

(c)  	Sound suppression fulfilling regulations regarding

Permissible operator noise levels in effect at the time

Of purchase.

(d)  	AM/FM stereo radio.

(e)  	Tinted glass throughout.

(f) 	Thermal insulation and floor mat.

(g) 	One inside mounted rear-view mirror.

(h) 	Dome light.

(i) 	Front windshield washer.

(j) 	Front and rear windshield wipers.

Page 2

New 2022, 4.25 cyd, front end loader bid specifications continued;

(k) 	Single place air suspension cloth seat adjustable for and aft,

And up and down with folding armrests and seat belt,

Manufacturer's best.

(l) 	Lockable doors.

(m) 	Two exterior mirrors (West Coast style) one left side,

one right side, both mirrors to be heated.

- Air conditioning factory installed no exterior roof mounts.

- Fender extensions, mud (front and rear) fenders shall cover width of tires.

MISCELLANEOUS:  The successful bidder is to furnish one (1) parts book including cross reference manual covering all parts not manufactured by the parent company.  One (1) service record manual, two (1) operators manuals, one (1) manual to include the schematics for hydraulic-air-electrical with the stipulation that 5% of the monthly lease payments will be withheld until the manuals are furnished. The following is a list of miscellaneous equipment to be furnished:

(a)	Low-pressure air alarm in cab.

(b)	Normal set of lighted engine monitoring gauges,

Including torque converter oil, air pressure, ammeter,

Water temperature, oil pressure, hour meter and fuel.

Console warning lights and buzzers required.

(c)	Horn.

(d)	Stop and taillights.

(e)	Direction turn signals with four way flashers.

(f)	Two cab mounted headlights and two rear mounted

Floodlights.

(g)	Anti-vandalism control group with lockable caps

For oil filler and dipstick, fuel tank, hydraulic tank

And radiator.

(h)	Engine compartment doors.

(i)	Locking toolbox to be mounted in place where it will

Not interfere with operation and servicing.

(j)	View bubble or dipstick for hydraulic oil level.

(k)	Electronic backup alarm.

(l)	Fuel tank shall have a minimum capacity of 75 U.S.

Gallons.

(m) Foot accelerator-decelerator pedal required.

Page 3

New 2022, 4.25 cyd, front end loader bid specifications continued;

(n)	Operating weight of loader without counterweights,

without loaded tires, but with ROPS cab, lubricants,

coolant, full fuel tank and operator, shall be a minimum

of 40,000 pounds.

(o)	Rear mounted counterweight of approximately 1,200 lbs.

(p)	Entire unit to meet applicable MIOSHA standards.

(q)	Access steps on both sides of machine, if available.

- Deliver loader to Crawford County Garage in Grayling Michigan.

- Warranty:  Full warranty for entire length of the lease.

- Automatic Ride Control.

- Rear View Camera.

Submit bids in a sealed envelope that is clearly marked with the words: "2022, 4.25cyd Front End Loader ".

“The Crawford County Road Commission, in accordance with Title VI of the Civil Rights Act of 1964, 78 Stat. 252, 42 USC 2000d to 2000d-4 and Title 49, Code of Federal Regulations, Department of Federally-assisted programs of the Department of Transportation issued pursuant to such Act, hereby notifies all bidders that it will affirmatively insure that in any contract entered into pursuant to this advertisement, minority business enterprises will be afforded full opportunity to submit bids in response to this invitation and will not be discriminated against on the grounds of gender, disability, race, color, or national origin in consideration for an award.”

Crawford County Road Commission reserves the right to reject any or all bids, to waive irregularities in the bid, to waive details in the specifications, to add or delete quantities or projects to or from the bid and to accept the bid deemed to be in the best interest of Crawford County.

- Board of Crawford County Road Commissioners

Ryan Halstead, Chairman

Gary Summers, Vice Chairman

- Cris Jones, Commissioner

Scott Hanson, Commissioner

Ronald Larson, Commissioner

Page 4

New 2022, 4.25 cyd, front end loader bid specifications continued;

- 2022, 4.25 cyd front end loader

- BID SHEET

(TO BE SUBMITTED WITH BID)

- Option

- Please indicate the additional cost to Bid Price for the following:

A factory installed quick coupler attachment, including, hydraulics, electrical controls, and a coupler frame, include in price the cost of pallet forks.

Option: Cost of quick coupler and pallet fork option.	$ ______________________

Bid Price (machine value)	$_______________________

Monthly payment.		$ ______________________

Monthly payment with quick coupler option added.

.	$ ______________________

Bid firm for: _____________days.                              _____________________________

Date:

___________________________________.              _____________________________

Delivery Date:	                                                  Representatives Signature

_______________________________________________________________________

Company Name	                               Phone#	       Fax#

________________________________________   ______________________________

Mailing Address                                                         Rep. Signature

________________________________________________________________________

City, State, Zip

Page 5